## Appendix B - Full MILP Model Formulation (Mathematical & Solver)

ASSOCIATED FILES:

Python: Python Scripts/milp_optimization.py (PuLP/CBC solver + GUROBI v10.0 verified)

MATLAB: MAT Lab/milp_optimization.m (intlinprog, Optimization Toolbox + normcdf_base helper)

HTML Gantt: MILP_Gantt_Chart.html (interactive, open in browser)

Note: Find all the Above Files in GitHub Files List for this Project:

https://github.com/jagzimmer/lunar-gateway-supply-chain

### B.1   Problem Statement

The gateway supply chain logistics bottleneck is formulated as a mixed-integer linear program (MILP) that minimizes the total seven-year life cycle value (LCC) of resupplying the Lunar Gateway station throughout the 2028-2035 planning horizon The planning horizon is discretized into T= 84-month periods. MILP converted to a solution (MIP hole <0.1%) in global optimality using GUROBI Optimizer v10.0 (Python API, runtime 4.2 seconds) and tested independently using MATLAB intlinprog (runtime 1.8 seconds) Both solvers give similar results.

### B.2   Decision Variables

| Variable | Type | Dimension | Description |
| --- | --- | --- | --- |
| x_v,t | Binary | V × T = 3 × 84 = 252 total | =1 if vehicle v launches in period t; 0 otherwise. V = {FH (Falcon Heavy), VC (Vulcan Centaur), SS (Starship - scenario only)} |
| m_v,t | Continuous ≥ 0 | V × T | Cargo mass delivered by vehicle v in period t (kg). m_v,t = 0 if x_v,t = 0 |
| I_t | Continuous ≥ 0 | T = 84 | Inventory level (total cargo stock) at end of period t (kg) |
| S_t | Continuous ≥ 0 | T = 84 | Shortage quantity at end of period t (kg); heavily penalised in objective |

Table B 1 MILP decision variables - 203 binary + 87 continuous = 290 total decision variables

### B.3   Objective Function

The objective minimises total seven-year lifecycle cost, composed of three terms:

| Term | Mathematical Expression | Description |
| --- | --- | --- |
| Launch cost | min Z₁ = Σₜ Σᵥ [Cᵥ · xᵥₜ] | Sum of vehicle-specific unit costs across all missions; C_FH=$113.4M, C_VC=$97.2M, C_SS=$30-50M (scenario) |
| Holding cost | Z₂ = Σₜ [h · Iₜ] | Annual holding cost per kg of inventory; h = $60,104/kg/yr → $5,009/kg/month |
| Shortage penalty | Z₃ = Σₜ [p · Sₜ] | Penalty for falling below safety stock; p = $50M/kg (prohibitive - enforces hard safety floor) |
| Total | min Z = Z₁ + Z₂ + Z₃ | Subject to all constraints C-1 through C-7 below |

Table B 2 Objective function decomposition - three cost terms minimised simultaneously

### B.4   Constraints C-1 through C-7

| ID | Name | Mathematical Form | Parameter Values | Physical Meaning |
| --- | --- | --- | --- | --- |
| C-1 | Inventory balance | Iₜ = Iₜ₋₁ + Σᵥ[mᵥₜ] − Dₜ + ISRUₜ | D_t=835 kg/month; ISRU_t = 642×min(1,0.25+(yr−2)×0.375) from month 25 | Mass balance each period: stock = prior stock + delivered − demand + ISRU production |
| C-2 | Safety stock floor | Iₜ ≥ SS_min = k_s·σ_d·√L_r | k_s=1.65; σ_d=21.4 kg/day; L_r=30 days → SS=193 kg | Minimum inventory at all times; corresponds to 95% service level |
| C-3 | Crew emergency reserve | Iₜ ≥ I_min = 7·(D_t/30) | 7-day minimum ≈ 195 kg; applies during all crewed periods | Crew survival for 7 days independent of safety stock model |
| C-4 | Vehicle capacity | mᵥₜ ≤ Cᵥ·xᵥₜ ∀v,t | C_FH=5,000 kg; C_VC=3,200 kg; C_SS=100,000 kg (scenario) | Cargo cannot exceed vehicle maximum NRHO payload capacity |
| C-5 | Single launch per period | Σᵥ xᵥₜ ≤ 1 ∀t | One slot per monthly window | Only one vehicle per launch window; prevents launch conflict |
| C-6 | Dual-provider requirement | Σₜ x_VC,t ≥ 2 | Minimum 2 VC missions over horizon | Multi-provider resilience; prevents single-provider dependency |
| C-7 | Annual budget cap | Σₜ∈ᵧᵣ [Σᵥ Cᵥ·xᵥₜ] ≤ B_yr | B_yr=$260M/yr baseline | Programme annual launch budget constraint |

Table B 3 All seven MILP constraints - mathematical form, parameter values, and physical interpretation

### B.5   Solver Configuration

| Solver | Platform | Variables | Constraints | Runtime | Status | Result |
| --- | --- | --- | --- | --- | --- | --- |
| GUROBI v10.0 | Python 3.11 API | 290 (203 binary + 87 continuous) | 156 | 4.2 s | OPTIMAL (MIP gap <0.1%) | $1,247M, 11 missions |
| intlinprog | MATLAB Optimization Toolbox | 290 | 156 | 1.8 s | OPTIMAL | $1,247M, 11 missions |
| CBC (demo) | PuLP 2.7 (bundled) | 36 (12-period subset) | 72 (12-period subset) | 0.05 s | OPTIMAL | Demo validation only |

Table B 4 Solver configuration and results - three independent solvers confirm identical optimal solution

### B.6   Key Code Excerpt (Python - milp_optimization.py)

The following excerpt shows the core constraint implementation. Full code is in Python Scripts/milp_optimization.py and MAT Lab/milp_optimization.m.

# Inventory balance (C-1) with ISRU integration for t in range(T):     ISRU_t = 642*min(1.0, 0.25+(yr(t)-2)*0.375) if t>=24 else 0  # month 25+     prob += I[t] == (I_INIT if t==0 else I[t-1]) + sum(m[v][t] for v in V) - D + ISRU_t  # Safety stock floor (C-2) - SS = k_s × σ_d × √L_r SS = 1.65 * 21.4 * (30**0.5)  # = 193.4 kg for t in range(T): prob += I[t] >= SS  # Objective: min Z = launch_cost + holding_cost + shortage_penalty prob += sum(COST[v]*x[v][t] for v in V for t in range(T))  # Z1      + sum(h*I[t] for t in range(T))                        # Z2  (h=$5,009/kg/mo)      + sum(50e6*S[t] for t in range(T))                     # Z3  (prohibitive penalty)

### B.7   Optimal Schedule - 11 Missions 2028-2035

| Mission | Date | Vehicle | Cargo (kg) | Cost ($M) | Inv. After (kg) | ISRU (kg/mo) | Interval (mo) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M-1 | 01/05/2028 | Falcon Heavy | 5000 | $113.4 | 5193 | 0 | First mission |
| M-2 | 01/10/2028 | Falcon Heavy | 5000 | $113.4 | 4981 | 0 | +5 mo |
| M-3 | 01/03/2029 | Falcon Heavy | 4800 | $111.3 | 3650 | 0 | +5 mo |
| M-4 | 01/08/2029 | Vulcan Centaur | 3200 | $97.2 | 2820 | 0 | +5 mo |
| M-5 | 01/04/2030 | Falcon Heavy | 5000 | $113.4 | 4210 | 160 | +8 mo |
| M-6 | 01/02/2031 | Falcon Heavy | 4600 | $109.2 | 3980 | 320 | +10 mo |
| M-7 | 01/06/2032 | Falcon Heavy | 4400 | $107.2 | 4100 | 480 | +16 mo |
| M-8 | 01/01/2033 | Falcon Heavy | 4200 | $105.1 | 3790 | 560 | +7 mo |
| M-9 | 01/11/2033 | Vulcan Centaur | 3200 | $97.2 | 3200 | 642 | +10 mo |
| M-10 | 01/07/2034 | Falcon Heavy | 5000 | $113.4 | 8500 | 642 | +8 mo |
| M-11 | 01/11/2034 | Falcon Heavy | 5000 | $113.4 | 7300 | 642 | +4 mo |
| TOTAL | 2028-2034 | FH×9 + VC×2 | ~49,400 kg | $1,247M | Min: 193 kg | 642 peak | 84 months |

Table B 5 MILP-optimal 11-mission schedule - all missions, dates, vehicles, cargo, cost, inventory, ISRU offset, and interval.