APPENDIX C

Complete FMEA Table - All 47 Failure Modes

FMEA SCOPE AND METHODOLOGY

Standard: MIL-STD-1629A Task 101 Failure Mode and Effects Analysis

RPN = Severity (S) × Occurrence (O) × Detectability (D) ¦ Each dimension: 1 (best) to 10 (worst)

Scope: Six supply chain functions (F-1 to F-6) + cross-system ICD interfaces ¦ Total: 47 failure modes

Priority threshold: RPN ≥ 80 = HIGH (mandatory mitigation) ¦ 4 failure modes exceed this threshold

Chapter 4 reference: Tables 10 RPNs (§4.1.3). This appendix documents all 47 modes.

Note: Find all the Above Files in GitHub Files List for this Project:

https://github.com/jagzimmer/lunar-gateway-supply-chain

Rating Scale Reference

| Score | Severity (S) | Occurrence (O) | Detectability (D) |
| --- | --- | --- | --- |
| 1–2 | Minor: negligible effect on mission or crew | Very unlikely: <1 event in 50 missions | Very detectable: automatically flagged before impact |
| 3–4 | Low: minor mission impact, no crew risk | Unlikely: ~1 event in 20 missions | Detectable: monitoring in place, usually caught early |
| 5–6 | Moderate: partial mission failure, limited crew risk | Occasional: ~1 event in 10 missions | Moderate: sometimes missed, requires inspection |
| 7–8 | High: mission failure or crew injury risk | Likely: ~1 event in 5 missions | Difficult: inconsistent detection, expert review needed |
| 9–10 | Critical: mission loss or crew safety/life risk | Very likely: >1 event in 5 missions | Very difficult: detection unlikely before effect occurs |

Table C 1 RPN rating scale - severity, occurrence, and detectability definitions (MIL-STD-1629A)

Complete FMEA - All 47 Failure Modes

Table C.1 presents all 47 failure modes identified across the six-supply chain functional areas (F-1 through F-6) and cross-system interface failures. Failure modes are ordered by RPN (highest to lowest) within each functional group. HIGH = RPN ≥ 80 (mandatory mitigation required per MIL-STD-1629A). The four failure modes exceeding this threshold (FM-01 to FM-04) correspond directly to the framework's highest-priority design interventions: dual-provider architecture (FM-01), backup docking port (FM-02), cold-chain redundancy (FM-03), and MILP-DST demand reforecast (FM-04).

UNCTION F-1: CARGO DEMAND PLANNING ¦ FM-01 to FM-10

| FM# | Function | Failure Mode | Effect | S | O | D | RPN | Priority | Recommended Action |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FM-01 | F-1 Demand | Launch vehicle primary failure — full cargo manifest lost | Mission loss; crew at risk; minimum 30-day resupply gap; potential emergency crew evacuation | 10 | 4 | 3 | 120 | HIGH | Dual-provider architecture (FH+VC); 90-day safety stock minimum; mission insurance per CER-7 risk model; emergency launch protocol |
| FM-02 | F-4 Transit | Dragon XL docking failure — cargo undeliverable at NRHO | Mission failure; $100M+ cargo inaccessible; launch slot wasted; crew supply at risk | 9 | 3 | 4 | 108 | HIGH | Backup NDS docking port; ground simulation validation pre-launch; contingency resupply within next window |
| FM-03 | F-2 Integration | Cold-chain failure — food/biologics temperature exceedance | Consumable loss; crew nutrition risk; emergency re-manifest; $8–15M loss | 8 | 4 | 3 | 96 | HIGH | Redundant thermal packaging; IoT in-flight temperature monitoring; 25% cold-chain mass margin; KSC cold storage backup |
| FM-04 | F-1 Demand | Demand forecast overestimate >20% — excess cargo manifested | Excess inventory carry cost $3–6M/yr; budget overrun; manifest inefficiency | 4 | 5 | 4 | 80 | HIGH | Rolling 6-month reforecast using MILP DST; ISS consumption data calibration; LHS Monte Carlo demand model |
| FM-05 | F-5 Receiving | HALO cargo storage pressurisation failure | Consumable contamination; crew safety risk; emergency O₂ reserve trigger | 9 | 2 | 4 | 72 | HIGH | Redundant sealing (leak-before-burst design); quarterly pressure integrity checks; 45-day emergency O₂ reserve |
| FM-06 | F-3 Ground | Canadarm3 software anomaly — cargo unhandled at Gateway | Delayed unloading; crew manual EVA handling required (risk); 3–5 day delay | 7 | 3 | 3 | 63 | HIGH | Software V&V per DO-178C; manual override procedure; pre-loaded autonomous stowage protocol |
| FM-07 | F-3 Ground | Ground cargo processing delay >21 days — launch window miss | 30-day resupply delay; crew supply buffer reduced below target | 6 | 5 | 2 | 60 | HIGH | VSM future-state interventions (§4.4); 45-day processing target; pre-positioned supplier inventory at KSC |
| FM-08 | F-6 ISRU | ISRU water extraction efficiency <50% (design margin violation) | Increased Earth-launched consumable demand; break-even delayed; $30–50M budget impact | 5 | 4 | 3 | 60 | HIGH | Conservative 75% design efficiency margin; phased deployment plan; annual efficiency audit; IME-1 pilot data |
| FM-09 | F-1 Demand | Demand forecast underestimate >15% — safety stock shortfall | Safety stock falls below 193 kg threshold; crew supply risk; emergency trigger | 6 | 4 | 3 | 72 | HIGH | Early-warning trigger at I_t < 1.2×SS; rolling demand model update; k_s=1.65 buffer absorbs ±15% demand variation |
| FM-10 | F-5 Receiving | Spares reliability model error — MTBF wrong by >50% | Incorrect spares quantity ordered; critical equipment failure without replacement part | 5 | 3 | 4 | 60 | HIGH | MIL-HDBK-217F reliability model; ISS actuals calibration; 20% spares growth allowance per mission |

FUNCTION F-2: CARGO MANIFEST & INTEGRATION ¦ FM-11 to FM-15

| FM# | Function | Failure Mode | Effect | S | O | D | RPN | Priority | Recommended Action |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FM-11 | F-2 Manifest | Cargo manifest data entry error — wrong items loaded | Incorrect items delivered; crew supply shortfall; costly return/replacement | 6 | 4 | 2 | 48 | MED | RFID 100% cargo tagging (Recommendation R2); automated manifest-to-stowage cross-check; pre-launch triple verification |
| FM-12 | F-2 Manifest | Launch mass budget overrun >5% — cargo offloading required | Cargo de-manifest; mission replanning; 5–10 day delay; science loss | 6 | 3 | 3 | 54 | MED | 15% mass contingency margin; daily mass tracking tool; 30-day manifest lock protocol; ICD-01 compliance check |
| FM-13 | F-2 Manifest | Volume constraint violation — cargo cannot be loaded into Dragon XL | Cargo rejected at pad; last-minute de-manifest; launch delay | 5 | 2 | 3 | 30 | LOW | ICD-02 volume verification tool; 3D manifest visualisation in DST; pre-integration volume simulation |
| FM-14 | F-2 Manifest | Hazmat classification error — incorrect UNCLASS or IATA code | Launch pad incident; cargo rejection; mission delay 5–21 days; regulatory penalty | 7 | 2 | 3 | 42 | MED | NASA Approved Hazmat Supplier Programme; dual-review classification; 21-day hazmat processing lead enforced |
| FM-15 | F-2 Manifest | Biological payload temperature exceedance pre-launch (KSC) | Payload spoilage; crew nutrition or science loss; $2–5M re-procurement | 6 | 3 | 3 | 54 | MED | Cold-chain monitoring (IoT sensors, 4°C ±2°C); KSC Payload Processing Facility cold storage; 14-day buffer re-supply slot |

FUNCTION F-3: GROUND PROCESSING & LAUNCH ¦ FM-16 to FM-22

| FM# | Function | Failure Mode | Effect | S | O | D | RPN | Priority | Recommended Action |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FM-16 | F-3 Ground | Propellant loading error — wrong propellant type or quantity | Mission failure; vehicle loss; launch abort; total cargo loss | 8 | 2 | 3 | 48 | MED | Dual-key propellant verification protocol; automated loading system verification; pre-launch propellant mass audit |
| FM-17 | F-3 Ground | Supplier late delivery — cargo components >7 days behind schedule | Processing delay; possible launch window miss; 30-day resupply delay | 4 | 4 | 3 | 48 | MED | Long-term supplier agreements per Recommendation R2; 14-day safety lead; approved supplier list with SLA |
| FM-18 | F-3 Ground | Weather delay exceeding launch window duration (2–3 day window) | Window miss; 30-day resupply delay; crew supply buffer reduced | 4 | 5 | 2 | 40 | MED | Dedicated Weather Assessment Team (KSC); 24-hr recycle window planning; staggered launch scheduling |
| FM-19 | F-3 Ground | KSC facility unavailability — competing launch priority conflict | Priority conflict; Gateway mission delayed; possible window miss | 3 | 3 | 3 | 27 | LOW | Reserved launch slot per contract; 60-day advance booking; alternative launch complex SLC-40 identified |
| FM-20 | F-3 Ground | Range safety system anomaly — launch abort triggered | Launch abort; 5–10 day recycle delay; launch costs partially sunk | 7 | 1 | 3 | 21 | LOW | Redundant range safety systems (primary + backup); pre-launch 48-hour range safety system test procedure |
| FM-21 | F-3 Ground | Pad fire or explosion during propellant handling | Vehicle loss; KSC facility damage; crew injury risk; mission failure | 10 | 1 | 2 | 20 | LOW | NFPA 30 compliance; automated fire suppression (CO₂ Halon); dedicated pad safety team; emergency egress |
| FM-22 | F-3 Ground | Ground cargo handling equipment failure at KSC | Processing delay; possible launch window miss; cargo damage risk | 4 | 3 | 3 | 36 | LOW | Preventive maintenance schedule; backup handling equipment on standby; 72-hour turnaround SLA with KSC |

FUNCTION F-4: IN-SPACE LOGISTICS ¦ FM-23 to FM-28

| FM# | Function | Failure Mode | Effect | S | O | D | RPN | Priority | Recommended Action |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FM-23 | F-4 Transit | Navigation GNC error during NRHO insertion manoeuvre | Mission failure; incorrect orbit insertion; fuel overuse; possible vehicle loss | 7 | 2 | 3 | 42 | MED | Autonomous GNC (NASA Deep Space Navigator); ground-monitored NRHO insertion; backup orbit correction window |
| FM-24 | F-4 Transit | Thermal control malfunction — cargo freezing during cislunar transit | Cold-sensitive cargo (biologics, batteries) damaged; $5–12M cargo loss | 6 | 2 | 4 | 48 | MED | Active thermal control system; redundant heaters; cargo thermal margin ±15°C verified pre-launch |
| FM-25 | F-4 Transit | Solar array partial failure — 30% power reduction | Reduced payload power; thermal management compromise; shortened mission | 6 | 2 | 3 | 36 | LOW | Redundant solar panel segments (3 of 4 sufficient); onboard power management software; 20% power margin |
| FM-26 | F-4 Transit | Orbital debris collision during cislunar transit | Vehicle loss: cargo lost; crew safety risk if crewed transit | 9 | 1 | 2 | 18 | LOW | Debris avoidance manoeuvre protocol; Space Surveillance Network TLE tracking; Dragon XL Whipple shield |
| FM-27 | F-4 Transit | ESPRIT module refuelling failure — propellant transfer incomplete | Reduced Gateway station-keeping ΔV; early deorbit risk if uncorrected | 5 | 2 | 3 | 30 | LOW | Manual propellant transfer backup procedure: 6-month extended station-keeping reserve; annual prop audit |
| FM-28 | F-4 Transit | O₂ generation system failure at Gateway (ECLSS primary) | Crew safety emergency; immediate abort/evacuation required | 10 | 2 | 2 | 40 | MED | Redundant ECLSS (primary + secondary electrolyser); 45-day O₂ emergency reserve; pre-defined crew abort protocol |

FUNCTION F-5: GATEWAY RECEIVING & STORAGE ¦ FM-29 to FM-33

| FM# | Function | Failure Mode | Effect | S | O | D | RPN | Priority | Recommended Action |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FM-29 | F-5 Receiving | Cargo stowage mis-sort — item stored in incorrect location | Crew time waste (avg 12-day recovery per VSM §4.4); mission efficiency loss | 3 | 6 | 3 | 54 | MED | RFID 100% cargo tagging; digital stowage manifest; automated location verification scan at stowage |
| FM-30 | F-5 Receiving | Water storage tank leak — potable water supply loss | Water supply reduction; crew safety risk; emergency resupply trigger at <14 days | 7 | 2 | 3 | 42 | MED | Redundant storage tanks (2 independent circuits); automated leak detection (pressure sensors); 30-day backup water reserve |
| FM-31 | F-5 Receiving | Food spoilage — expired or contaminated due to storage failure | Crew nutrition risk; manifest re-supply required within 30 days | 6 | 2 | 4 | 48 | MED | Shelf-life tracking system (RFID integrated); 6-month rotation protocol; 25% food reserve margin per mission |
| FM-32 | F-5 Receiving | Crew injury during cargo transfer EVA — musculoskeletal or suit failure | Mission delay; medical evacuation risk; crew incapacitation | 7 | 2 | 3 | 42 | MED | Canadarm3 autonomous cargo assist (planned 2028); EVA training updates; ICD-03 grapple fixture verification |
| FM-33 | F-5 Receiving | Fire in pressurised module — cargo packaging as ignition source | Crew safety emergency; module evacuation; cargo loss; station damage | 10 | 1 | 3 | 30 | LOW | Smoke detection throughout; automated CO₂ suppression; non-combustible packaging standards; fire containment bags |

FUNCTION F-6: ISRU OPERATIONS ¦ FM-34 to FM-38

| FM# | Function | Failure Mode | Effect | S | O | D | RPN | Priority | Recommended Action |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FM-34 | F-6 ISRU | ISRU drilling/excavation equipment mechanical failure | ISRU output halved; Earth supply demand increases by ~300 kg/month | 5 | 3 | 3 | 45 | MED | Redundant drilling heads (2× primary); modular replacement design; 30% ISRU capacity margin in MILP model |
| FM-35 | F-6 ISRU | Regolith chemistry deviation — water ice content lower than model | ISRU yield below 75% target; break-even delayed; additional Earth resupply | 4 | 4 | 3 | 48 | MED | Conservative 75% efficiency model (sensitivity analysis §4.5.1); pilot ISRU test pre-full deployment (NASA IME-1) |
| FM-36 | F-6 ISRU | ISRU water contamination — perchlorate from regolith processing | Water unsafe for crew; purification required; production halt 3–7 days | 7 | 2 | 3 | 42 | MED | Electrolysis purification system with perchlorate filter; real-time water quality monitoring; bypass to Earth-supplied water |
| FM-37 | F-6 ISRU | ISRU electrolysis failure — O₂ production loss | Crew O₂ shortfall; emergency Earth-supply trigger; ECLSS backup required | 7 | 2 | 3 | 42 | MED | Redundant electrolyser cells (primary + secondary); 45-day O₂ emergency reserve; ECLSS primary-backup auto-switch |
| FM-38 | F-6 ISRU | ISRU power supply loss — solar/battery system failure | ISRU offline; Earth supply demand increases; break-even delayed | 6 | 2 | 3 | 36 | LOW | Dedicated ISRU power system (10 kW solar array); battery backup 72-hour capacity; manual ISRU restart procedure |

CROSS-SYSTEM INTERFACE FAILURES ¦ FM-39 to FM-47

| FM# | Function | Failure Mode | Effect | S | O | D | RPN | Priority | Recommended Action |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FM-39 | F-1/F-2 Interface | NASA-ESA-JAXA-CSA partner approval delay — single largest NVA (VSM) | 28-day average NVA (§4.4.1 Table 4.13); manifest freeze; delayed launch | 4 | 5 | 2 | 40 | MED | Single-gate approval process (Recommendation R2 §6.3.2); pre-agreed monthly decision matrix; delegated authority levels |
| FM-40 | F-5/ICD-06 | HALO-Gateway communications failure — Lunar Link relay outage | Crew-ground disconnect; autonomous ops required; decision authority unclear | 5 | 3 | 3 | 45 | MED | Lunar Link backup relay (secondary satellite); 72-hour autonomous ops protocol; onboard stored procedures |
| FM-41 | F-1/F-2 DST | Software version mismatch — DST manifest tool incompatibility | Manifest error propagation; re-planning delay 3–5 days; cost impact | 3 | 4 | 4 | 48 | MED | Version-controlled DST deployment; formal change control (CCB); regression test suite before each release |
| FM-42 | F-5 Crew | Crew psychological stress — extended isolation (Gateway intermittent) | Decision-making error; mission risk increase; medical intervention required | 5 | 2 | 4 | 40 | MED | Crew rotation schedule ≤90 days; psychological support protocol; real-time DSN video comms 96% availability |
| FM-43 | ICD-06 | Cybersecurity breach — manifest or mission data exfiltration/corruption | Manifest corruption; supply chain attack; mission data compromise | 5 | 1 | 4 | 20 | LOW | Zero-trust network architecture; air-gapped manifest system; NASA CSEC cybersecurity compliance; annual pen-test |
| FM-44 | F-4 ICD-02 | CLPS surface mission failure — ISRU precursor hardware undelivered | ISRU commissioning delayed 12–18 months; break-even shifts to Year 5.5; $30–82M impact | 7 | 4 | 2 | 56 | HIGH | Multi-CLPS provider strategy; non-critical payload routing only to CLPS; backup FH cargo slot within 6 months |
| FM-45 | F-3 Ground | Cargo manifest recall post-launch window lock — late change request | Re-manifest penalty; 5–10-day processing delay; possible window miss | 3 | 3 | 3 | 27 | LOW | 60-day manifest lock protocol; change request freeze 45 days pre-launch; DST automated impact assessment |
| FM-46 | F-5 ICD-03 | Canadarm3 grapple fixture failure — unpressurised cargo unhandled | External cargo stranded; crew EVA required; 3–5-day mission delay | 7 | 2 | 3 | 42 | MED | Pre-flight ICD-03 grapple fixture verification; backup manual EVA contingency procedure; spare fixture stored |
| FM-47 | F-2 Manifest | CLPS manifest over-commitment — surface payload exceeds lander capacity | Surface mission failure or partial delivery; science loss; schedule slip | 5 | 3 | 2 | 30 | LOW | ICD integration review 90 days pre-launch; Gateway-CLPS integrated manifest tool; mass margin ≥15% |

Table C 2 Complete FMEA - all 47 failure modes per MIL-STD-1629A. HIGH = RPN ≥ 80. Ordered by RPN within each functional group.

FMEA Summary Statistics

| Function | FM Range | Count | Max RPN | HIGH (≥80) | Key Mitigation |
| --- | --- | --- | --- | --- | --- |
| F-1: Cargo Demand Planning | FM-01 to FM-10 | 10 | 120 (FM-01) | 3 (FM-01,03,04) | Dual provider: MILP DST reforecast; safety stock k_s=1.65 |
| F-2: Cargo Manifest & Integration | FM-11 to FM-15 | 5 | 54 (FM-12,15) | 0 | RFID tagging; mass contingency; manifest lock protocol |
| F-3: Ground Processing & Launch | FM-16 to FM-22 | 7 | 48 (FM-16,17) | 0 | VSM lean interventions; supplier SLA; weather assessment team |
| F-4: In-Space Logistics | FM-23 to FM-28 | 6 | 48 (FM-24) | 0 | Redundant GNC; thermal control; CLPS multi-provider |
| F-5: Gateway Receiving & Storage | FM-29 to FM-33 | 5 | 54 (FM-29,31) | 0 | RFID stowage; water redundancy; Canadarm3 autonomous assist |
| F-6: ISRU Operations | FM-34 to FM-38 | 5 | 48 (FM-35) | 0 | Redundant drilling; pilot test; water purification; 30% margin |
| Cross-System Interfaces | FM-39 to FM-47 | 9 | 56 (FM-44) | 1 (FM-44) | Single-gate approval; Lunar Link backup; version control DST |
| TOTAL | FM-01 to FM-47 | 47 | 120 (FM-01) | 4 (RPN≥80) | Post-mitigation average RPN reduction: 43.7% |

Table C 3 FMEA summary statistics by functional area - count, maximum RPN, high-priority count, and primary mitigation theme