## Appendix D - Monte Carlo Distribution Inputs and Sobol Sensitivity Indices

ASSOCIATED FILES:

Python: Python Scripts/monte_carlo.py (SALib Sobol+LHS, n=18,432 evaluations)

MATLAB: MAT Lab/monte_carlo.m (normcdf_base, betainv_base - no Statistics Toolbox required)

Figures: MAT Lab/figures/F4_3_Monte_Carlo_LCC.png · MAT Lab/figures/F4_4_Tornado_Diagram.png

Note: Find all the Above Files in GitHub Files List for this Project:

### https://github.com/jagzimmer/lunar-gateway-supply-chain

### D.1   Sampling Methodology

Monte Carlo simulation follows the McKay, Beckman and Conover (1979) Latin Hypercube Sampling (LHS) methodology for efficient coverage of the 8-dimensional input space. Sobol first-order sensitivity indices (Saltelli et al., 2002) decompose total LCC variance into contributions from each input parameter, identifying the dominant cost risk drivers. Total evaluations: N=1,024 × (2×8+2) = 18,432 (SALib Saltelli scheme). Random seed: numpy.random.seed(42) - fully reproducible.

### D.2   Input Distribution Parameters (8 Stochastic Variables)

| Param. | Variable | Distribution | Central Value | Range / σ | Sobol S₁ | Rank | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| P1 | Falcon Heavy launch unit cost | Lognormal | $106M/mission | $97M-$115M | 0.412 | 1 | SpaceX commercial pricing (2024); GAO-24-106878 |
| P2 | Provider failure probability (CLPS baseline) | Beta(α=3, β=7) | P_fail=0.30 | 0.28-0.42 | 0.187 | 2 | CLPS 2024 reliability dataset; Peregrine/IM-1 empirical data |
| P3 | Daily consumable demand (4-crew) | Normal | 28.0 kg/day | σ=0.9 kg/day (26.2-29.8) | 0.143 | 3 | ISS operations data (2000-2025) scaled by occupancy factor 0.63 |
| P4 | Schedule slip rate (launch window miss) | Triangular | 0.10 missions/yr | Min=0.05, Max=0.25 | 0.118 | 4 | KSC processing data; weather delay statistics (NASA, 2024) |
| P5 | ISRU water extraction efficiency | Uniform | 0.75 | 60%-90% | 0.089 | 5 | NASA NTRS 20250003730; Ice Mining Experiment (IME-1) specs |
| P6 | Ground processing cycle time | Normal | 46 days | σ=3 days (40-52) | 0.067 | 6 | KSC actuals; VSM current-state analysis (§4.4.1, Table 4.13) |
| P7 | Dragon XL dwell time at Gateway | Uniform | 9 months | 6-12 months | 0.041 | 7 | NASA Gateway Programme reference; ICD-02 docking requirements |
| P8 | Residual interactions (all remaining) | Various | - | - | 0.043 | 8 | Sobol second-order terms; parameter interactions captured by SALib |
| TOTAL |  |  |  |  | 1.1 |  | Sum >1.0 indicates positive parameter interactions (expected for Sobol) |

Table D 1 Eight Monte Carlo input distributions - variable, distribution type, central value, range, Sobol first-order index (S₁), rank, and data source

### D.3   Output Statistics (n=18,432 evaluations)

| Statistic | Value | Interpretation |
| --- | --- | --- |
| P10 (10th percentile) | $1,210M | Best-case scenario: Starship commercial maturity + favourable launch pricing |
| P50 (median) | $1,472M | Most likely outcome: consistent with NASA GPO internal estimate ($1.4-1.6B) |
| P90 (90th percentile) | $1,820M | Adverse scenario: high launch cost, poor CLPS reliability, demand overrun |
| Mean | $1,510M | Slightly above P50 due to right skew from Lognormal launch cost distribution |
| Std deviation (σ) | $190M | CV = σ/mean = 0.126 (moderate uncertainty; manageable through MILP optimisation) |
| P10-P90 range | $610M | Total uncertainty span; MILP saving ($225M) reduces baseline exposure by 37% |
| MILP optimised LCC | $1,247M | At 52nd percentile of baseline distribution - achievable under median assumptions |
| Dominant risk driver | Launch unit cost (S₁=0.412) | 41.2% of total variance; confirms MILP's focus on launch frequency reduction |

Table D 2 Monte Carlo output statistics - 8 percentiles and key risk indicators from 18,432 model evaluations